COMPETITION-GRADE

PID LINE FOLLOWER ROBOT

Complete Technical Documentation

Arduino Nano  •  ATmega328P @ 16MHz  •  TB6612FNG  •  QTR-8A

WS2812 RGB  •  HC-05 Bluetooth  •  2S Li-Po

Firmware Rev 1.0  —  March 2026

# 1. Project Overview

This document covers the full hardware specification, firmware architecture, PID control theory, calibration procedure, Bluetooth command protocol, and competition tuning guide for the Arduino Nano-based PID line follower robot.

The robot is designed for competition use on a black tape track (1.5 cm line width) on a white surface. The primary design goal is reliable handling of sharp 90° turns using a hybrid differential/pivot control strategy, while maintaining maximum straight-line speed.

## 1.1 Feature Summary

| Feature | Implementation | Notes |
| --- | --- | --- |
| Controller | Arduino Nano (ATmega328P) | 16 MHz, 5V logic |
| Sensor array | QTR-8A (6 of 8 active) | Analog, position-weighted |
| Motor driver | TB6612FNG | Dual H-bridge, 1.2A cont. |
| Drive motors | 2× N20 DC gear motor | 12V rated, 50:1 gear ratio |
| Turn strategy | Hybrid PID + full pivot | Threshold: ¦error¦ > 350 |
| Speed profiling | Automatic decel on curves | 70% speed at pivot threshold |
| Calibration | Auto-spin + EEPROM save | Triggered via BT or button |
| PID persistence | EEPROM (update-safe) | Survives power cycle |
| Lap timer | ms precision, BT report | Start/stop with RUN/U |
| LED feedback | 8× WS2812 NeoPixel | State + line position display |
| Wireless tuning | HC-05 Bluetooth @ 9600 | Full PID + speed + color |
| Battery monitor | 47k/10k divider on A5 | Color coded low-battery warn |

# 2. Hardware Reference

## 2.1 Pin Assignment Map

| Pin | Signal | Direction | Connected To |
| --- | --- | --- | --- |
| A0 | Sensor S3 | INPUT | QTR-8A out 3 (4th from left) |
| A1 | Sensor S0 | INPUT | QTR-8A out 0 (leftmost) |
| A2 | Sensor S1 | INPUT | QTR-8A out 1 |
| A3 | Sensor S2 | INPUT | QTR-8A out 2 |
| A5 | Battery ADC | INPUT | 47k/10k voltage divider |
| A6 | Sensor S5 | INPUT | QTR-8A out 5 (rightmost) |
| A7 | Sensor S4 | INPUT | QTR-8A out 4 |
| D0 | BT TX→RX | INPUT | HC-05 TX (hardware Serial) |
| D1 | BT RX←TX | OUTPUT | HC-05 RX via voltage divider |
| D2 | BTN UP | INPUT | Button, ext. pull-up, active LOW |
| D3 | BTN SELECT | INPUT | Button, ext. pull-up, active LOW |
| D4 | BTN DOWN | INPUT | Button, ext. pull-up, active LOW |
| D6 | PWMB | OUTPUT | TB6612 right motor PWM (Timer0) |
| D7 | BIN1 | OUTPUT | TB6612 right motor dir A |
| D8 | BIN2 | OUTPUT | TB6612 right motor dir B |
| D9 | PWMA | OUTPUT | TB6612 left motor PWM (Timer1) |
| D10 | AIN1 | OUTPUT | TB6612 left motor dir A |
| D11 | AIN2 | OUTPUT | TB6612 left motor dir B |
| D12 | RGB DATA | OUTPUT | WS2812 data line |
| 5V | Emitter EN | OUTPUT | QTR-8A emitter (always ON) |
| 5V | STBY | OUTPUT | TB6612 standby disable (always ON) |

## 2.2 Sensor Physical Ordering

The QTR-8A sensor array pins are not wired in sequential analog pin order. The firmware maps them in physical left-to-right order:

| Physical Position | Label | Arduino Pin | PID Weight |
| --- | --- | --- | --- |
| 1 (leftmost) | S0 | A1 | -500 |
| 2 | S1 | A2 | -300 |
| 3 | S2 | A3 | -100 |
| 4 | S3 | A0 | +100 |
| 5 | S4 | A7 | +300 |
| 6 (rightmost) | S5 | A6 | +500 |

NOTE The sensor emitter pin is wired directly to 5V so it stays permanently enabled. If you need power-saving, drive it from a digital pin and add a pinMode OUTPUT + digitalWrite HIGH call in setup().

## 2.3 Power System

| Rail | Source | Voltage Range | Consumers |
| --- | --- | --- | --- |
| VM (motor) | 2S Li-Po direct | 7.4V – 8.4V | TB6612 VM, motor coils |
| 5V (logic) | Buck converter | 5.0V regulated | Nano VIN, QTR, WS2812, HC-05 |
| Battery ADC | 47k / 10k divider | 0 – 5V scaled | A5 pin (factor ×5.7) |

WARNING Never feed raw 2S Li-Po to the Arduino Nano VIN. Always use the 5V buck converter rail. The Nano's onboard regulator cannot handle 8.4V input reliably at competition motor loads.

Battery divider calculation:   Vbat = ADC_value × (5.0 / 1023) × 5.7

| LED Color | Voltage Range | Status |
| --- | --- | --- |
| Green | > 7.8V | Good — race-ready |
| Orange | 7.2V – 7.8V | Medium — consider recharging between rounds |
| Red | < 7.2V | Low — recharge before racing |

# 3. Firmware Architecture

## 3.1 State Machine

The robot operates as a three-state machine. All state transitions are triggered by Bluetooth commands or physical button presses.

| State | LED Color | Motors | Entry Trigger | Exit Trigger |
| --- | --- | --- | --- | --- |
| IDLE | Blue (solid) | Off | Boot / Stop cmd / Line lost | RUN cmd / SELECT btn |
| CALIBRATING | Orange (solid) | Slow spin | Q cmd | Auto (6s complete) |
| RUNNING | User color | PID control | RUN cmd / SELECT btn | U cmd / SELECT btn / Line lost 2s |

## 3.2 Main Loop Execution Order

Each loop() iteration executes in this fixed priority order:

- handleBT() — drain Serial RX buffer, parse complete commands

- handleButtons() — debounced edge detection on 3 buttons

- computeError() — read 6 ADC channels, compute weighted position error

- pidCompute() — PID math, select mode (differential or pivot), output motor speeds

- motorLeft() / motorRight() — write PWM + direction to TB6612

- runningLEDs() — update WS2812 to reflect current line position

- Telemetry TX — send E:/L:/R: over BT at 10Hz (non-blocking timer check)

- Line loss watchdog — hard stop after 2000ms with no sensor active

NOTE There are no blocking delays anywhere in the main loop. The calibration routine is the only blocking section and it runs outside RUNNING state. This ensures BT commands and button presses remain responsive at all times.

## 3.3 PWM Timer Configuration

Timer prescalers are reconfigured in setup() to push PWM frequency above the audible range, which also reduces EMI coupling into analog sensor reads:

| Timer | Pins Affected | Default Freq | Configured Freq | Side Effects |
| --- | --- | --- | --- | --- |
| Timer1 | D9 (PWMA), D10 | 490 Hz | 31.25 kHz | None — Timer1 not used by core libs |
| Timer2 | D11 (AIN2), D3 | 490 Hz | 31.25 kHz | None — D3 is button input, not PWM |
| Timer0 | D6 (PWMB) | 976 Hz | Unchanged | Shared with millis()/delay() — left as-is |

# 4. PID Control System

## 4.1 Position Error Computation

Error is derived from a weighted centroid of all 6 normalized sensor readings. This gives a continuous position estimate rather than discrete bang-bang switching.

### Step 1 — Normalize each sensor

norm[i] = (raw[i] - calMin[i]) / (calMax[i] - calMin[i])  ×  1000    → range: 0 (white) to 1000 (black)

### Step 2 — Weighted centroid

error = Σ(norm[i] × weight[i])  /  Σ(norm[i])

Where weights are:

| Sensor | S0 | S1 | S2 | S3 | S4 | S5 |
| --- | --- | --- | --- | --- | --- | --- |
| Weight | -500 | -300 | -100 | +100 | +300 | +500 |

Error = 0 → line perfectly centred under array

Error < 0 → line is to the LEFT of centre → steer left

Error > 0 → line is to the RIGHT of centre → steer right

## 4.2 PID Output Calculation

correction = Kp × error  +  Ki × integral  +  Kd × derivative

derivative = error - lastError

integral   = clamp(integral + error,  -5000,  +5000)

Default values:   Kp = 0.25  Ki = 0.0002  Kd = 8.0

NOTE The integral clamp of ±5000 prevents windup on long straight sections. The integrator is also hard-reset to 0 on every entry into pivot mode, preventing the accumulated straight-line integral from causing overshoot on exit from a sharp turn.

## 4.3 Hybrid Turn Strategy

The firmware divides error magnitude into three zones, each with a different motor output strategy:

| Zone | ¦error¦ Range | Strategy | Speed | Notes |
| --- | --- | --- | --- | --- |
| Tracking | 0 – 200 | PID differential | 100% baseSpeed | Normal line following |
| Decel ramp | 200 – 350 | PID differential | 100% → 70% (interpolated) | Pre-corner braking |
| Pivot | 350 – 600+ | Full pivot turn | 100% baseSpeed | Integrator reset, reverse inner wheel |

### Pivot Mode Motor Outputs

| Condition | Left Motor | Right Motor | Result |
| --- | --- | --- | --- |
| error > 0 (line right) | +baseSpeed | -baseSpeed | Pivot clockwise — robot turns right |
| error < 0 (line left) | -baseSpeed | +baseSpeed | Pivot counter-clockwise — robot turns left |

TIP If the robot oscillates after exiting a 90° turn, the pivot is completing too fast or undershooting and triggering pivot in the opposite direction. Lower PIVOT_THRESHOLD from 350 to 280, or increase Kd by 2–3 to damp the exit.

## 4.4 Line Loss Recovery

When all 6 sensors read below the active threshold (norm < 200 per sensor), the robot is considered off-line. Recovery behavior:

- computeError() returns ±600 in the direction of the last known line position

- pidCompute() sees |error| > PIVOT_THRESHOLD → immediately enters pivot mode toward last-known direction

- Robot spins in place until sensors re-acquire the line

- If line is not recovered within 2000ms, motorsStop() is called and state → IDLE

# 5. Calibration Procedure

## 5.1 What Calibration Does

Calibration samples the raw ADC range for each sensor over a 6-second auto-spin sequence. The measured minimum (white) and maximum (black) values for each sensor are stored in EEPROM. These are used to normalize sensor readings to a 0–1000 range that is independent of ambient light, surface reflectivity variation, or LED aging.

## 5.2 Auto-Spin Sequence

| Time | Motor Action | Purpose |
| --- | --- | --- |
| 0 – 1.5s | Spin left (slow) | Sweep all sensors over black and white |
| 1.5 – 4.5s | Spin right (slow) | Reverse sweep for full coverage |
| 4.5 – 6.0s | Spin left (slow) | Return toward start position |

NOTE You can also manually slide the robot across the track during calibration instead of relying on the auto-spin. The firmware just takes min/max continuously for the full 6 seconds regardless of movement source.

## 5.3 Calibration Steps

- Place robot on the track with sensors centered over the line

- Send Q via Bluetooth (or add Q to your start sequence)

- LEDs turn orange — robot begins auto-spin

- Wait 6 seconds until LEDs turn green

- BT responds with CAL_DONE

- Calibration values auto-saved to EEPROM — persists across power cycles

WARNING Re-calibrate every time you change surface, lighting conditions, or sensor height. A badly calibrated robot will drift off-line even with perfect PID values.

## 5.4 EEPROM Memory Map

| Address | Size | Type | Content |
| --- | --- | --- | --- |
| 0x00 | 4B | float | Kp |
| 0x04 | 4B | float | Ki |
| 0x08 | 4B | float | Kd |
| 0x0C | 2B | int | baseSpeed |
| 0x0E | 1B | byte | Magic byte (0xAB = valid data) |
| 0x10 | 12B | int[6] | calMin[0..5] (2B each) |
| 0x1C | 12B | int[6] | calMax[0..5] (2B each) |

EEPROM writes use EEPROM.update() which only physically writes if the value has changed. This protects flash endurance (rated 100,000 write cycles) during frequent tune sessions.

# 6. Bluetooth Command Protocol

## 6.1 Physical Layer

| Parameter | Value |
| --- | --- |
| Module | HC-05 |
| Baud rate | 9600 |
| Data bits | 8 |
| Stop bits | 1 |
| Parity | None |
| Terminator | \n or \r (either accepted) |
| TX pin | D0 (Nano hardware Serial RX) |
| RX pin | D1 via voltage divider (5V → 3.3V for HC-05) |

## 6.2 Command Reference

| Command | Format | Action | Response |
| --- | --- | --- | --- |
| RUN | RUN | Start robot, reset PID, start lap timer | None (LED changes to user color) |
| STOP | U | Stop motors, print lap time, go IDLE | LAP:xxxx ms |
| CAL | Q | Run 6s auto-calibration routine, save to EEPROM | CAL_DONE |
| BATT | B | Read battery voltage, show color on LEDs | None |
| READ | R | Print current PID + speed values | KP:x KI:x KD:x SPD:x |
| SET Kp | P<value> | Set proportional gain, save EEPROM | None |
| SET Ki | I<value> | Set integral gain, save EEPROM | None |
| SET Kd | D<value> | Set derivative gain, save EEPROM | None |
| SPEED | S<value> | Set base motor speed 0–255, save EEPROM | None |
| COLOR | C <R> <G> <B> | Set WS2812 running color (0–255 each) | None |

EXAMPLE To set Kp=0.30, Ki=0.0003, Kd=10.0, speed=170, send these commands in sequence: P0.30 I0.0003 D10.0 S170

## 6.3 Telemetry Stream

While running, the firmware transmits live telemetry at 10Hz (every 100ms):

E:<error>  L:<leftPWM>  R:<rightPWM>

Example:   E:47 L:182 R:136

Use any Bluetooth serial terminal (Serial Bluetooth Terminal on Android, CoolTerm on PC) to monitor this stream and plot the error signal in real time.

# 7. LED Status Reference

## 7.1 System State Colors

| Color | Pattern | Meaning |
| --- | --- | --- |
| Blue (solid) | All 8 LEDs | IDLE — waiting for RUN command |
| Orange (solid) | All 8 LEDs | CALIBRATING — do not move robot manually |
| Green (solid) | All 8 LEDs | Calibration complete / battery good |
| User color | 3 LEDs moving | RUNNING — line position indicator active |
| Red (solid) | All 8 LEDs | Line lost — robot has auto-stopped |

## 7.2 Running Mode — Line Position Display

During RUNNING state, 3 LEDs show the estimated line position. The brightest LED represents the sensor closest to the line. The two adjacent LEDs illuminate at 25% brightness as context:

| LEDs Lit (brightest) | Line Position |
| --- | --- |
| LED 0 (leftmost) | Line is far left — hard left correction active |
| LED 1–2 | Line is left of centre |
| LED 2–3 | Line is near centre — normal tracking |
| LED 3–4 | Line is right of centre |
| LED 5 (rightmost) | Line is far right — hard right correction active |

# 8. Competition Tuning Guide

## 8.1 Recommended Tuning Sequence

Always tune in this order. Do not jump ahead — each step assumes the previous is stable.

| Step | Action | Goal | BT Commands |
| --- | --- | --- | --- |
| 1 | Calibrate | Accurate normalized sensor readings | Q |
| 2 | Set conservative start | Robot tracks slowly but stably | P0.20 I0.0001 D5.0 S120 |
| 3 | Increase Kp until oscillates | Find oscillation threshold on straights | P0.30 P0.35 etc. |
| 4 | Back off Kp 20% | Stable tracking without oscillation | e.g. `P0.28` |
| 5 | Increase Kd | Dampen oscillation, allow higher Kp | D8.0 D10.0 D12.0 |
| 6 | Add small Ki | Eliminate steady-state drift on long straights | I0.0002 I0.0003 |
| 7 | Increase base speed | Find max speed that still handles 90° turns reliably | S150 S170 S190 |
| 8 | Test pivot threshold | If 90° turns miss: lower threshold. If jittery: raise it. | Edit PIVOT_THRESHOLD in code |

## 8.2 Symptom → Fix Table

| Symptom | Likely Cause | Fix |
| --- | --- | --- |
| Oscillates on straights | Kp too high | Reduce Kp by 15–20% |
| Slow to correct on curves | Kp too low or Kd too high | Increase Kp, reduce Kd |
| Overshoots after 90° turn | Integrator windup / Kd too low | Integrator auto-resets on pivot; increase Kd |
| Misses 90° turn entirely | PIVOT_THRESHOLD too high or speed high | Lower threshold to 280; reduce base speed |
| Jitters at straight-line centre | Kd too high | Reduce Kd by 1–2 steps |
| Drifts to one side on straight | Ki too low or sensors need recal | Recalibrate; add Ki 0.0001 |
| Robot spins and can't recover | Line loss with wrong lastDir | Recalibrate; check sensor min/max are sensible |
| Sluggish on straights | Base speed too low or speed profile | Increase S; SPEED_PROFILE_FACTOR is already 0.70 |
| Works in test but fails at comp | Lighting / surface difference | Recalibrate on comp surface, under comp lighting |

## 8.3 Key Tunable Constants

| Constant | Location | Default | Effect |
| --- | --- | --- | --- |
| DEFAULT_KP | Define | 0.25 | Proportional gain starting value |
| DEFAULT_KI | Define | 0.0002 | Integral gain starting value |
| DEFAULT_KD | Define | 8.0 | Derivative gain starting value |
| DEFAULT_SPEED | Define | 160 | Base PWM speed on first boot |
| PIVOT_THRESHOLD | Define | 350 | ¦error¦ above which pivot activates |
| MAX_INTEGRAL | Define | 5000 | Anti-windup clamp on integrator |
| SPEED_PROFILE_FACTOR | Define | 0.70 | Speed at pivot threshold entry (fraction) |
| DEBOUNCE_MS | Define | 30 | Button debounce period in ms |

# 9. Physical Button Reference

| Button | Pin | Single Press Action | EEPROM Save |
| --- | --- | --- | --- |
| UP | D2 | Increase base speed by 5 (max 255). Reports new speed via BT. | Yes |
| SELECT | D3 | Toggle RUNNING ↔ IDLE. Starts/stops lap timer. Reports lap time on stop. | No |
| DOWN | D4 | Decrease base speed by 5 (min 50). Reports new speed via BT. | Yes |

All buttons use external pull-up resistors and are active LOW. The firmware uses non-blocking debounce with a 30ms window — no delay() calls involved.

# 10. Required Libraries

| Library | Source | Purpose |
| --- | --- | --- |
| EEPROM | Arduino built-in | Persistent parameter storage |
| Adafruit NeoPixel | Arduino Library Manager | WS2812 LED control |

Install Adafruit NeoPixel via Arduino IDE: Sketch → Include Library → Manage Libraries → search "Adafruit NeoPixel" → Install.

# 11. Quick Start Checklist

- Install Adafruit NeoPixel library in Arduino IDE

- Flash linefollower_pid.ino to Arduino Nano

- On first boot: LEDs go blue, BT prints BOOT_OK + default PID values

- Pair HC-05 at 9600 baud (default PIN: 1234 or 0000)

- Place robot on track, send Q — wait for CAL_DONE

- Send RUN — robot begins following

- Monitor E:/L:/R: telemetry, tune Kp/Ki/Kd via BT

- Send U to stop — lap time reported automatically

TIP Keep a tuning log. After each session send R to read current values and record them. EEPROM saves automatically but having a written log lets you recover a known-good tune after a bad change.

Competition PID Line Follower  —  Technical Documentation  —  Rev 1.0  —  March 2026